TL70 Programmable Audible Programming Instructions

- Remove the top cover to the TL70 Programmable Audible Module, by rotating the top cover counterclockwise. Connect the USB cable to the computer and to the Audible Module as shown below.

Notes:

- During programming, external power must be removed from the Audible Module.

- The USB to Mini USB cable is sold separately, Banner Part #USB-USBM-1.

- Open the program by clicking on “Start, All Programs, Banner Audio” and the main programming window will open.

Note:

- If the program is started without the USB cable or the Audible Module, you will see this error. This is just a warning letting you know that the device is not connected. You can click “OK” to proceed and the program will operate as normal.

- Click on “Add File” and point the program to where the .wav or .mp3 files are stored.

- Select the file you want to program into the Audible Module and then click “Open”.

Note:

- The file length cannot be more than 130 seconds. Continuous single tones of 2KHz or above may distort due to sampling rate. For best results uses files with 11KHz sample rates.

- Click on “Write” to transfer the file into the indicator.

Note:

- Any existing file previously loaded into the Audible Module will be erased prior to writing the new file.

- The software will erase any previously written file and write the selected file into the indicator. The status window will show “SPI programming”. This only takes a few seconds.

- If you receive the following error, then your file is too large or longer than the 130-second limit. Reduce the size or length of your file and try again.

- Once the programming is completed, the status window will say “Write Success”.

- Disconnect the Mini USB cable from the module, and apply power to the TL70 module to listen to the new file. The audible will play the new file continuously until the power is removed from the device.

- See the TL70 Programmable Audible datasheet, for additional information.